Nebraska Department of Transportation Mix Design

Jacob Kosmicki, Ryder Anderson, Mehgan

Dr. Lane

Course: CIVE 202

2/2/2026

Client: Nebraska Department of Transportation

Contents

Table of Figures	2

Introduction	3

Methods	3

Mix Design	5

References	9

# Table of Figures

Table 1 Excel to Python table	4

Table 2 Mix designs populated from code	6

# Introduction

Concrete is a fundamental part of construction in the modern age. It is used throughout construction. Concrete is comprised of cementitious materials, water, and aggregate. The portioning of these materials directly affects the durability, workability, and strength of the final concrete product. The Nebraska Department of Transportation (herein then referred to as NDOT) currently has an Excel document handling this process for easy computing of the formulas.

The main goal of this project is to translate the Excel logic into a Python code base. The reasons for this are firstly to improve the reliability of the calculations. Secondly, the calculations being in Python mean they are fixed, transparent, and repeatable. Lastly, the code being in Python it makes it more suitable for web-based integration if NDOT chooses to pursue that route in the future.

The primary objectives of this report are:

- Explain the process of translating Excel mix design logic into Python functions.

- Evaluate four mix designs within the code.

- Discussing four mix designs tested within Python.

# Methods

All analysis and functions was conducted in a. ipynb file. The panda library was used for data handling. Matpitib was used for printing graphical outputs for visual processing. The reference material used was the NDOT mix design document, specifically the Mix Design tab. This document defines the calculation logic used in this project. The NDOT mix design process follows an absolute volume approach, meaning all mix components must sum to 27 cubic feet (one cubic yard). Volume calculations use water’s unit weight of 62.4 lb/ft^3.

The Excel document was analyzed to determine any dependencies. The table below outlines the variables, their description, the Excel logic and the Python formula.

Table 1 Excel to Python table

| Variable | Description | Excel Formula (Logic) | Python Implementation |
| --- | --- | --- | --- |
| Q | Weight of water (lb) | = (Cement + Fly Ash + Silica Fume + SCM) × W/C | Q = (A+B+C+D) * E |
| R | Volume of cement (ft³) | = Cement Weight / (SG_cement × 62.4) | R = A / (J*62.4) |
| S | Volume of fly ash (ft³) | = Fly Ash Weight / (SG_flyash × 62.4) | S = B / (K*62.4) |
| T | Volume of silica fume (ft³) | = Silica Fume Weight / (SG_sf × 62.4) | T = C / (L*62.4) |
| U | Volume of other SCM (ft³) | = SCM Weight / (SG_scm × 62.4) | U = D / (M*62.4) |
| V | Volume of air (ft³) | = (Air%) / 100 × 27 | V = (F/100) * 27 |
| W | Volume of water (ft³) | = Water Weight / 62.4 | W = Q / 62.4 |
| X | Volume of total aggregate (ft³) | = 27 – R – S – T – U – V – W | X = 27-R-S-T-U-V-W |
| Y | Weight of fine aggregate (lb) | = 62.4 × (Fine%) × SG_fine × X | Y = 62.4*(G/100)*N*X |
| Z | Weight of coarse aggregate (lb) | = 62.4 × (Coarse%) × SG_coarse × X | Z = 62.4*(H/100)*O*X |
| AA | Weight of other aggregate (lb) | = 62.4 × (Other%) × SG_other × X | AA = 62.4*(I/100)*P*X |

The code collects the input values from the user, these included the specific gravity and the target design amounts. The functions output values that constitute the weight chart for one cubic yard of concrete.

# Mix Design

Using the code detailed above four separate mix designs were ran. Mix #1 was High-Strength 6,000 psi Silica-Fume-Modified Concrete Mix, Mix #2 was Standard DOT 4,000 psi Structural Concrete Mix, Mix #3 was High Replacement SCM Sustainable Concrete Mix, the mast mix, Mix #4 was Lightweight Structural Concrete Mix (ASTM C330 Aggregates). These four mixes are the concrete types we tested.

Mix 1 is a high-strength concrete used in high-performance applications (Sika AG, n.d.). It can be used for high load applications such as bridges or can be used for structural repair, excelling at deep-pour applications specifically. Some of its characteristics or advantages are exhibiting proper freeze/thaw resistance, and its ability to be used in almost every structural repair, including repairs to slab on grade, above grade and below grade concrete conditions and for repairs to vertical and overhead surfaces.

Mix 2 is a Standard DOT 4,000 psi Structural Concrete Mix (Quikrete, n.d.). This mix represents a standard concrete use that is applicable to almost all infrastructure, including roadways, storm systems such as culverts, and retaining walls. Some characteristics that this mix exhibits are good bonding to steel, high durability while also being high strength, and due to its high usage, it is a cost-effective option. This mix was chosen to give the client something that they might be familiar with due to it’s applications in roadway infrastructure.

Mix 3 is a High Replacement SCM Sustainable Concrete Mix (Chahar & Pal, 2025). This mix utilizes a high volume of supplementary cement material such as fly ash and slag in place of the normal Portland cement. This allows this mix to lower it’s carbon footprint released by curing by removing the mass of the Portland cement while maintaining a high strength yield, typically above 2,500 psi, letting it maintain structural integrity.

Mix 4 is a Lightweight Structural Concrete Mix also known as ASTM C330 (US Made Supply, n.d.). This mix exhibits a lower density than conventional mixes by specifying a lower specific gravity for the fine aggregates. The main objective of this mix is to lower the deadload of concrete structures such as bridges, while maintaining a high strength of 2,500 psi. Another application of this mix is that lightweight concrete exhibits a higher fire-resistance compared to normal-weight concrete, striving to potentially lower costly fireproofing.

Table 2 Mix designs populated from code

| Component | Mix 1 (High-Strength 6k psi) | Mix 2 (Standard 4k psi DOT) | Mix 3 (High SCM Sustainable) | Mix 4 (Lightweight ASTM C330) |
| --- | --- | --- | --- | --- |
| Cement (lb) | 650.00 | 520.00 | 350.00 | 560.00 |
| Fly Ash (lb) | 100.00 | 120.00 | 200.00 | 80.00 |
| Silica Fume (lb) | 40.00 | 0.00 | 0.00 | 0.00 |
| Other SCM (lb) | 0.00 | 0.00 | 150.00 | 0.00 |
| Fine Aggregate (lb) | 1,106.10 | 1,145.47 | 1,180.22 | 1,314.76 |
| Coarse Aggregate (lb) | 1,690.45 | 1,604.74 | 1,517.42 | 1,363.10 |
| Other Aggregate (lb) | 142.79 | 140.48 | 137.85 | 139.64 |
| Water (lb) | 252.80 | 288.00 | 294.00 | 281.60 |
| W/C Ratio (water/cement) | 0.389 | 0.554 | 0.840 | 0.503 |
| Air Content (%) | 4% | 6% | 5% | 7% |

The four mix designs produced valid results consistent with their intended applications. They provided a useful range of scenarios for validating the code.

Mix 1 exhibits the highest total cement content at 790 lb/yd³. This along with it also having the highest coarse aggregate weight of 1,690 lb and lowest water content of 252.8 lb shows it is consistent with its high strength design parameters. Also, with the inclusion of 40 lbs of silica fume, it exhibits a denser matrix by filling the pore space in between particles.

Mix 2 is a standard DOT mix which is represented by it balanced approach. With a water-to-cement ratio of 0.45 along with a solid freeze-thaw resistance, this mix is particularly useful for Nebraska’s climate. These characteristics, along with a total cement content of 640 lb/yd³, this mix reflects standard DOT specifications.

Mix 3 reflects the main goal of High Replacement SCM Sustainable Concrete Mix with its cement content being 350 lb/yd³. This is replaced with 200 lb of fly ash and 150 lb of slag, yielding a total cementitious content of 700 lb/yd³. The water-to-cement ratio computed is 0.84, which appears high at first glance. Further investigation, however, leads to the function that computes water apart of the total cementitious material. Upon further investigation, the total water came to be 294 lb, which is a more reasonable unit when weighed against the total cementitious content. This is an important note to keep in mind while computing heavy SCM mixtures.

Mix 4 yielded the highest fine aggregate weight of 1,314.76 lb and the lowest coarse aggregate weight of 1,363.10 lb among the four mixes. The lower fine aggregate specific gravity of 2.55, when compared to the other 2.65 used by the other mixes, caused the volume to be slightly lower in weight. This is consistent with the design intent of this mix.

Across the four mixes, the automated Python code successfully reproduced the calculations from the NDOT worksheet provided by the client. The mixes that are used were sought out in the goal of using diverse figures in each mix. With the code successfully producing the correct figures, it confirms that the Python translation from the Excel is robust and usable.

# References

Sikacrete®-360 SCC Sika AG. (n.d.). Sikacrete®-360 SCC. Sika. Sikacrete®-360 SCC

4000 PSI Concrete Mix | Strength, Uses & Benefits Quikrete. (n.d.). 4000 PSI concrete mix: Strength, uses & benefits. Quikrete. 4000 PSI Concrete Mix | Strength, Uses & Benefits

Chahar, A. S., & Pal, P. (2025). Effects of sustainable materials on high-performance concrete. Discover Applied Sciences, 7, 1110. https://doi.org/10.1007/s42452-025-07535-8

ASTM C330 Lightweight Aggregate: Bulk Density Requirements & Guide | US MADE SUPPLY US Made Supply. (n.d.). ASTM C330 lightweight aggregate: Bulk density requirements & guide. https://usmadesupply.com/blogs/news/astm-c330-lightweight-aggregate